Biomimetic Water-Surface Robot with Automatic Obstacle Avoidance

Team "Set Sail", Course Project, South China University of Technology

Role: Mechanical design & control implementation (team of 5)
Tools: SolidWorks, 3D printing (resin & PLA), Arduino UNO, C/C++, ultrasonic & color sensors

Project Brief

Inspired by the remarkable locomotion of water striders, we designed and built a surface-swimming robot that can move forward/backward, turn in place, and automatically avoid obstacles in a pool environment. The robot "walks" on water using surface tension and distributed buoyancy, and autonomously reaches a target region while maintaining stability.

Mechanical Design

Water-strider-inspired structure:

Four "legs" with adjustable foam floats at their tips provide buoyancy and stability.

The body is modeled in SolidWorks with an arched upper frame, base, legs, floating plates and a compact propulsion module.

Chebyshev linkage propulsion:

Each side uses a Chebyshev linkage driven by a TT geared DC motor to transform rotary motion into a combined up-down & back-forth stroke, mimicking the paddling motion of a water strider.

The geared motors provide sufficient torque under low supply voltage and keep the mechanism compact.

Material choice & iterations:

Early prototypes used LEGO components for rapid exploration; final versions combined resin (body) and PLA (gearing) for better rigidity and waterproofing.

Structural symmetry and reconfigurable floating plates were used to tune balance and draft depth.

Electronics & Control

On-board controller: Arduino UNO + L298N motor driver controlling two TT geared motors.

Motion control:

Bidirectional motor control via H-bridge; PWM is used to equalize left/right speed and correct heading drift.

Autonomous behaviors:

Obstacle avoidance: Ultrasonic sensor measures distance; when an obstacle is within a threshold, the robot executes a search routine: rotate while scanning until a collision-free direction is found.

Target stopping: A color sensor reads the RGB value of the pool bottom; when a predefined color patch is detected, the robot brakes and stops at the goal region.

Experimental Results

Tested in a 2.61 m x 1.72 m pool with different water depths and obstacles.

The robot successfully demonstrated all required behaviors: move forward/backward, turn left/right, and autonomously avoid a fixed obstacle while traversing from one side of the pool to the other.

The final prototype showed good stability but revealed improvement directions in propulsion efficiency and weight distribution, which we documented for future iterations.

My Contributions & Learning

Led the mechanism design and SolidWorks modeling, including the Chebyshev linkage and floating-leg assembly.

Implemented and tuned PWM-based speed control and the obstacle-avoidance logic on Arduino.

Conducted systematic pool tests, diagnosed issues in direction drift and imbalance, and iterated the mechanical and control design.

Through this project I built a complete workflow from bio-inspired concept -> SolidWorks -> mechatronic integration -> field testing, and gained hands-on experience with over-water robotics and sensor-based autonomous control.

Climbing-Assisted Hand Exoskeleton
Design and Manufacturing II Course Project, South China University of Technology

Project Overview
In this team project, we designed and built a wearable hand exoskeleton that assists climbers in maintaining a secure grip and prevents accidental slip during climbing. The device is worn like a glove and can automatically lock the fingers around a hold when needed, then release them when it is safe to move. To make the system reliable in safety-critical scenarios, we adopted a four-layer architecture--sensing, control, actuation and safety--so that each function can be designed, tested and debugged independently.

Thin-film pressure sensors placed near the finger joints continuously measure contact forces on the holds. These signals are processed by an Arduino-based controller running a "three-point contact" algorithm: only when the system judges that the climber still has enough contact points does it unlock one hand for motion; otherwise it enforces a locked, supporting grip. A high-torque serial bus servo drives a spatial four-bar linkage on the back of the hand, giving about 2.5x mechanical advantage while keeping the glove module under 0.5 kg.

To guarantee fail-safe behavior, we designed a bidirectional ratchet locking mechanism that can mechanically hold the fingers at any angle even if power is lost. Dual emergency-stop mechanisms (a hardware kill switch and a software watchdog) and a "transparent mode" (near-zero resistance when no support is needed) further improve comfort and safety.

Technical Highlights

Four-layer system architecture - Sensing (pressure sensor array), control (Arduino + bus-servo driver at 100 Hz), actuation (HX08L bus servo + four-bar linkage), and safety (mechanical ratchet + dual e-stops).

Biomechanics-inspired linkage - Spatial four-bar mechanism tuned to match human finger flexion, designed through motion-stroke analysis and force/moment calculation to support a 16 N load on a 25 mm bar.

Bidirectional passive lock - Dual ratchet wheels and spring-loaded pawls hold the finger rigidly in both flexion and extension; a cable-driven release restores free motion on demand.

Embedded sensing & logic - Thin-film RP-L-110 sensors with thresholding and a simplified three-point-contact rule decide when it is safe to unlock a hand for the next move.

Manufacturing and integration - PLA FDM-printed structural parts with metal reinforcement at high-load regions, CNC-milled aluminium ratchets, custom wiring harness and PCB for power and UART bus communication.

My Contributions

Co-designed the mechanical architecture of the glove module, including the four-bar linkage geometry and initial force/stroke calculations for the finger actuation.

Helped design the bidirectional ratchet mechanism for passive locking, focusing on tooth profile, pawl layout and integration with the linkage.

Implemented and debugged the Arduino-side control loop, including sensor acquisition, lock/unlock state machine and communication with the bus-servo driver.

Participated in fabrication and assembly: 3D printing and post-processing of PLA parts, basic CNC and hand-assembly of the linkage and ratchet, and wiring of sensors and servos.

Designed and ran bench tests (lock time, stroke completion, load holding) and assisted with climbing-rig trials, then analyzed the experimental data to compare against our design targets.

Results

Response speed: Full lock achieved in approximately 0.18 s, faster than our 0.20 s design target.

Load capacity: Held a 16 N suspended load on a 25 mm bar with essentially zero slip over repeated 10 s trials.

Wearability: Glove module mass ~ 449 g; arm-mounted electronics < 1 kg; full system with battery < 1.5 kg.

Dexterity: Independent actuation and locking of thumb, index and middle fingers without noticeable cross-coupling.

Safety in climbing simulation: In 2-minute continuous runs on a 75 degrees climbing rig, the system achieved >90% successful grasp transitions, showed no structural deformation, and the passive lock arrested deliberate "failure" events of a single hand.

This project demonstrates my ability to design and analyze complex mechanisms, integrate sensors and embedded control, and validate a safety-critical wearable robot through systematic experimentation.

Multifunctional Ball-Collecting Vehicle

Remote-Controlled Mecanum-Wheel Robot for Ball Pickup Competition

Time / Context
2023-2024 Design & Manufacturing II, South China University of Technology
Team project (4-5 students) - omni-directional ball-collecting robot constrained to a 50 x 50 x 50 cm volume for a school competition.

Project Overview

We designed and built a compact, collision-based ball-collecting robot capable of picking up and placing balls ranging from ping-pong balls to basketballs. The robot had to:

Fit entirely inside a 50 cm cube while carrying its full mechanism.

Collect balls from vertical poles and transport them into a scoring zone within a limited time.

Survive repeated impacts and side-collisions in a highly dynamic contest environment.

Our final prototype used Mecanum wheels for full holonomic motion, a passive collision-driven pickup mechanism instead of a complex gripper, and a custom suspension system to protect electronics and maintain stability. The robot won the silver medal in the university ball-collecting robot competition.

System Design

Mechanical Design

Developed an impact-based pickup mechanism: the robot drives into the pole so that the ball is knocked down and, by inertia and a sloped guide plate, automatically falls into an internal storage bin--no articulated arm or vision system required.

Designed the chassis and body in SolidWorks, using an aluminum-profile frame and nylon panels to balance stiffness, crash resistance, and weight.

Implemented a rear gate + deployable barrier for controlled unloading: opening the gate lets balls roll out by gravity while a retractable lower barrier keeps them within the scoring zone.

Added a spring-based suspension system at each wheel to attenuate vertical oscillation from the Mecanum wheels and protect the bottom-mounted control boards and batteries during collisions.

Locomotion & Actuation

Four Mecanum wheels driven by 24 V planetary gear motors with encoders, enabling lateral translation, in-place rotation, and precise alignment with poles.

High-torque digital servo (~20 kg cm) to actuate the rear gate for fast, repeatable ball release.

Electronics & Control

STM32-based core board (C50C) with integrated encoder interfaces and multiple motor/servo channels.

PS2 wireless controller for intuitive remote operation during the contest.

Implemented PID speed control using encoder feedback to maintain stable wheel speeds and accurate omnidirectional motion; used PWM control for motor and servo outputs.

My Contributions

Led the concept and trade-off study using Quality Function Deployment (QFD): translated "low cost, crash resistance, ball capacity, ease of operation, and aesthetics" into engineering targets such as ball-picking efficiency, chassis stiffness, motor torque, and material choices.

Co-designed the pickup and storage geometry in CAD, iterating the slope angle and entry height so that balls of different diameters reliably fell into the storage area after impact.

Participated in suspension and frame design, selecting spring stiffness and mounting geometry to reduce vibration while keeping the overall height within the 50 cm limit.

Implemented and debugged key parts of the embedded control code (motor PID, Mecanum kinematics mapping, input smoothing, and safety limits on PWM to prevent hardware damage).

Helped with integration testing and on-field tuning, adjusting PID gains, gear ratio selection, and collision speed to maximize ball pick-up success rate while avoiding mechanical damage.

Technical Highlights & Outcomes

Innovative collision-based collection: eliminated a full manipulator and vision system, simplifying mechanics and electronics while remaining robust to ball size variation.

Highly maneuverable platform: Mecanum drive + suspension achieved smooth lateral approaches to poles and precise orientation in tight contest spaces.

Robust construction under repeated impacts: nylon-aluminum hybrid body absorbed shocks without structural failure throughout the competition.

Competition result: successfully completed the pick-and-place tasks and earned the silver medal in the school's ball-picking robot contest, demonstrating both engineering reliability and effective control.

Embedded Smart Home Terminal with Lightweight Audio Recognition
STM32 + TinyML + IoT Cloud + LLM Voice Assistant

Overview
This project builds a low-power smart-home terminal that performs on-device speech recognition on an STM32 microcontroller and only connects to the cloud when necessary. The goal is to replace purely cloud-based voice assistants with a privacy-preserving, low-latency system that can run on constrained hardware while still supporting natural, continuous dialogue through a large language model.

System Architecture
The terminal combines an STM32F4-Discovery board with two ESP32 modules (ESP32-WROOM and ESP32-C3), a TFT-LCD screen, an INMP441 digital microphone, audio amplifier, and speaker. The STM32 handles real-time signal processing and inference; the ESP32-C3 exposes Wi-Fi and MQTT connectivity to the OneNET IoT cloud, and the ESP32-WROOM connects to the iFlytek Spark large language model (LLM) for online dialogue.

On the device, I implemented a TinyML pipeline:

Collected and labeled my own Mandarin voice commands (e.g., "turn on the light", "open the fan").

Performed MFCC feature extraction and trained a small neural network classifier.

Deployed the model to the STM32 with TensorFlow Lite Micro, keeping both RAM and Flash within the board's tight limits while maintaining reliable command recognition.

Recognized commands are mapped to smart-home actions and sent via MQTT to the OneNET cloud; a simple HBuilderX-based web interface subscribes to these messages so lights, fans, and other virtual devices can be controlled and visualized in real time. The ESP32 LLM client streams audio to the iFlytek Spark backend and returns text responses, enabling continuous voice chat, weather queries, and other open-ended interactions.

My Contributions

Designed and implemented the full embedded speech pipeline: data collection, MFCC processing, neural-network training, and deployment with TensorFlow Lite Micro on STM32.

Integrated STM32-ESP32 communication and MQTT-based control via OneNET, allowing the terminal to orchestrate multiple smart-home devices under strict compute and memory budgets.

Implemented the LLM access flow on ESP32 to connect with the Xinghuo (iFlytek Spark) model for context-aware, multi-turn voice dialogue.

Outcome & Takeaways
The prototype achieves responsive offline command recognition on a bare-metal MCU while seamlessly escalating to a cloud LLM for richer conversations, demonstrating a practical path toward privacy-aware, low-cost smart-home interaction. Through this project I deepened my experience in embedded C/C++, TinyML deployment, IoT protocol design, and system-level co-design between hardware constraints, machine-learning models, and user experience.

Camera-Free Deformation Reconstruction of Soft Robots via Flexible Sensor Array
ISURE Summer Research Program, University of Notre Dame
Jul 2025 - Sep 2025 | Advisor: Prof. Tingyu Cheng | Role: Undergraduate Researcher & Co-author (ICRA 2026 submission)

Project Overview
This project builds a camera-free, real-time "digital twin" for soft robots. By combining a low-cost flexible piezoresistive sensor array with a 4D Gaussian Splatting (4DGS) deformation model and a cage-based graph neural network, we reconstruct the full 3D shape of a deformable silicone robot in real time using only tactile signals--no cameras or markers at test time.

Technical Approach

Flexible Sensor & Soft Robot Fabrication - Designed and fabricated a 3 cm x 3 cm, 10x10 flexible pressure sensor patch using silver-plated nylon electrodes, Velostat piezoresistive film, and TPU encapsulation. Integrated the patch onto custom silicone soft-robot bodies and implemented an ESP32 + multiplexers readout circuit for 100-channel, 200-500 Hz resistance sensing with real-time filtering and calibration.

4D Gaussian Supervision Pipeline - Built the deformation ground-truth pipeline: structured-light 3D scanning of ~10 key poses per motion (bend, twist, press), multi-view rendering in Blender (100 viewpoints per pose), video-frame interpolation (x8-x16 with RIFE), and 4DGS training to obtain temporally trackable Gaussian primitives for each frame.

Cage-based GNN Deformer - Implemented a graph-attention-network (GAT) deformer that maps the 10x10 resistance image and time encoding to low-dimensional cage-node displacements. Bound millions of Gaussians to the cage and propagated motion via inverse-distance-weighted interpolation on GPU, enabling scalable deformation prediction at interactive rates.

Real-Time Rendering & Interaction - Integrated a Gaussian-splatting rasterizer to render deformed Gaussians at 1280x800-1600x1000 resolution, with alpha compositing, depth sorting, and EMA temporal smoothing. Built a small UI to control viewpoint, field of view and smoothing for teleoperation-style monitoring.

My Contributions

Led sensor hardware iteration: layout design of the flexible pressure matrix, assembly and encapsulation process, two-point per-channel calibration, and physical integration onto soft-robot specimens.

Implemented the data-acquisition and preprocessing stack: ESP32 firmware, synchronized logging of resistance frames with 4DGS deformation data, temporal/spatial filtering, and normalization.

Co-implemented the GNN cage-deformation network in PyTorch, including sensor CNN encoder, time/cage positional encodings, GAT layers, and sparse cage-to-Gaussian propagation kernels on GPU.

Set up and tuned the 4DGS supervision pipeline (Blender rendering scripts, frame interpolation, training configuration), and ran all ablation and quantitative evaluations.

Results & Impact

Achieved ~30 FPS real-time rendering in a coarse setting and maintained <5 % mean 3D displacement error between reconstructed and ground-truth shapes.

Demonstrated <4.7 degrees bending-angle error under bending/twisting motions and stable trajectories under pressing, verified by synchronized camera images vs. reconstructed views.

Showed zero-shot camera-free reconstruction on unseen objects (e.g., a wallet) by reusing the same trained deformer with only a new static Gaussian model and sensor footprint.

Co-authored a manuscript, "Real-Time Camera-Free Deformation Reconstruction via Flexible Sensor Array and Cage-based Deformation Model," submitted to ICRA 2026.

Skills Demonstrated
Flexible electronic sensor design and fabrication; soft-robot prototyping; 3D scanning and Blender rendering; 4D Gaussian Splatting; graph neural networks (GAT) in PyTorch; GPU optimization; experimental design, error analysis, and research-paper writing.

Wine Quality Prediction with Ensemble Machine Learning

基于集成学习的葡萄酒质量预测研究（含 IPMLP 2025 论文工作）

Role & Context
Independent research project stemming from my Machine Learning course at UCLA, later extended into a single-author ACM paper "Wine Quality Prediction with Ensemble Trees: A Unified, Leak-Free Comparative Study" (accepted to IPMLP 2025). Supervised by Prof. Chunyang Liao (UCLA).

Problem & Motivation

Professional wine scoring is expensive and subjective, yet the underlying aroma and taste are determined by measurable physicochemical indicators such as acidity, sulphates and alcohol content. I set out to build a rigorous, reproducible machine-learning pipeline that can:

Predict wine quality from lab measurements alone.

Handle highly imbalanced, multi-class labels (quality scores 0-10).

Compare modern ensemble methods under strict, leak-free evaluation, and derive a compact set of key chemical indicators.

The project uses the canonical Vinho Verde red and white wine datasets from the UCI repository (1,599 red and 4,898 white samples, each with 11 physicochemical variables plus a sensory quality rating).

Technical Approach

1. End-to-end leak-free pipeline

To avoid the "optimistic results" common in prior work, I designed a strict experimental protocol:

Stratified 80/20 train-test split, with the 20% test set kept completely untouched until final reporting.

5-fold StratifiedGroupKFold on the training set to preserve label ratios and group related samples, preventing near-duplicates from leaking across folds.

In each training fold only:

StandardScaler normalization.

SMOTE-Tomek hybrid resampling to oversample rare quality levels and remove ambiguous boundary samples.

Cost-sensitive training with inverse-frequency class weights.

All preprocessing parameters and synthetic samples are confined within each training fold; validation and test sets retain their original distributions.

This pipeline enforces a clean separation between training, validation and test, and provides unbiased generalization estimates for all models.

2. Ensemble models & hyperparameter search

I benchmarked five state-of-the-art gradient-tree ensembles: Random Forest, Gradient Boosting, XGBoost, LightGBM and CatBoost.

Used Optuna TPE with median pruning for hyperparameter optimization (120-200 trials per model), tuned inside the 5-fold cross-validation only.

Training ran on an AMD EPYC 9K84 + NVIDIA H20 node with up to eight parallel workers, giving me experience in orchestrating large-scale experiments and managing GPU memory constraints.

3. Two-stage feature selection

To understand which physical variables really matter and to reduce sensing cost, I implemented a two-stage importance-driven feature selection scheme:

Train each model on all 11 features and compute normalized importance.

Select the top 5 variables per model (with a minimum 5% importance threshold) and re-train.

Across models, the key variables consistently converged to:

Red wine: alcohol, volatile acidity, sulphates, chlorides, and (for most models) total sulfur dioxide.

White wine: alcohol, free sulfur dioxide, chlorides, plus fixed acidity in place of some red-wine features.

4. Broader price-quality analysis

Beyond the UCI dataset, I also used a Kaggle rating-and-price dataset to explore economic implications:

Confirmed that higher-rated wines generally command higher prices.

Defined a simple "value metric" = rating / price to surface wines that provide the best quality-for-money trade-off.

Results & Insights

On the full 11-feature input, Gradient Boosting achieved the best test performance with weighted F₁ = 0.693 ± 0.028 (red) and 0.664 ± 0.016 (white).

Random Forest and XGBoost formed a strong second tier, staying within 3 percentage points of the leader in weighted F₁ while often matching or exceeding its macro AUC.

Compressing each model to its top 5 features reduced weighted F₁ by only 2.6 points (red) and 3.0 points (white) on average--roughly a 6-7% relative drop despite a 55% reduction in feature dimension, showing that a compact set of five easily measured variables is enough for high-fidelity predictions.

The SMOTE-Tomek + cost-sensitive training scheme reduced the class imbalance ratio from about 20.7 -> 1.04 (red) and 82.9 -> 1.02 (white), substantially improving macro F₁ and MCC without inflating test error.

Runtime analysis showed a clear cost-accuracy trade-off:

Gradient Boosting: ~12 hours per full five-fold study (highest accuracy).

XGBoost / LightGBM: 2-3 hours with GPU.

CatBoost: ~1 hour.

Random Forest: ~50 minutes on CPU with weighted F₁ ~ 0.65 and macro AUC ≥ 0.86, making it the most cost-effective production model.

These findings provide both a performance ranking (Gradient Boosting as the accuracy ceiling; Random Forest/XGBoost as practical workhorses) and a reproducible baseline that other researchers can build on.

My Contributions & Skills Demonstrated

Designed and implemented the entire leak-free pipeline (data splitting, resampling, class-weighted losses, feature selection and evaluation).

Wrote all experiment code in Python (scikit-learn, XGBoost, LightGBM, CatBoost, Optuna), automated hundreds of trials, and maintained experiment logs for reproducibility.

Performed error analysis (confusion matrices, minority-class behavior) and interpreted feature-importance patterns in terms of physical chemistry.

Authored the full ACM paper--from framing the problem and reviewing literature to writing, figures, and responding to reviews--strengthening my academic writing and experimental design skills.

This project shows my ability to treat machine learning as an engineered system: specifying clear objectives, building a robust pipeline, quantifying trade-offs between accuracy and cost, and translating technical results into actionable guidance for real-world decision-making.

Real-Time Robotic Arm Motion Control from Temporal Logic
Student Research Program (SRP), Human-Robot Interaction & Task Planning

Role: Undergraduate researcher (2-person team)

Advisor: Prof. Gang Chen, Shien-Ming Wu School of Intelligent Engineering, SCUT

Duration: Apr 2024 - Mar 2025

Tech Stack: Python, ROS2, MoveIt, Gazebo, rclpy, Linear Temporal Logic (LTL/GR(1))

Project Overview

Industrial manipulators are still hard to program for multi-step tasks with safety and ordering constraints ("first pick A, then B, always avoid region C"). In this SRP project I built a formal-methods-driven control pipeline where the user specifies the task in Linear Temporal Logic (LTL) and the system automatically:

Checks whether the specification is realizable with the robot's current skills and workspace.

Repairs unrealizable specifications by modifying the task formula.

Generates collision-free trajectories in ROS2 / MoveIt that provably satisfy the repaired temporal-logic task.

The goal was to bridge human-intuitive logical task descriptions and real-time robotic motion planning for human-robot interaction.

Methodology & System Design

Temporal-Logic Task Specification

Users describe tasks as LTL formulas, e.g. "eventually visit region A and then B, while always avoiding region D".

I translate these formulas into GR(1)-style automata that encode ordering, safety and liveness constraints, enabling algorithmic analysis and synthesis.

Two-Stage "Symbolic + Physical" Repair Loop

Based on the Synthesis-Based Repair framework, I implemented a two-level planner:

Symbolic repair:

Encode each robot skill (e.g. go-to(region), grasp(object)) and the overall task as LTL.

Run realizability checks; when the task is impossible (e.g. due to missing skills or unsafe paths), automatically pinpoint conflicting subformulas.

Repair the task by inserting intermediate steps, relaxing ordering, or adding guard conditions, while preserving the user's high-level intent.

Physical feasibility check:

For each symbolic plan, call MoveIt to synthesize a collision-free trajectory.

On failure (e.g. unreachable pose, collision with obstacles), convert the failure reason into new logical constraints and feed it back to the symbolic repair stage.

This closed loop continues until we obtain a task specification that is both logically valid and physically executable.

ROS2 / MoveIt Integration

Built ROS2 nodes (in rclpy) that subscribe to high-level actions from the temporal-logic planner and call MoveIt to execute each step on a simulated industrial arm.

Implemented asynchronous execution: the high-level planner keeps running while low-level motions execute, with completion and failure signals fed back via ROS topics and services.

Added continuous monitors for "always" constraints (e.g. always avoid a forbidden zone); violation triggers immediate replanning and symbolic repair.

Representative Scenarios

In Gazebo + MoveIt, I demonstrated tasks such as:

Sequential pick-and-place: "pick the red block, then the blue block, in order, without entering a dangerous region."

Skill-deficit repair: deliberately remove a required skill so the original LTL task is unrealizable; the system automatically augments the specification (e.g. inserting a detour waypoint) and recovers a feasible plan.

Dynamic interaction (prototype): mid-task changes to the specification (new sub-goal or new obstacle) trigger re-synthesis and replanning without restarting the system.

My Contributions

Conducted an extensive literature review and reproduced three state-of-the-art baselines (LCRL, E-LDGBA-LTL, and Synthesis-Based Repair), experimentally evaluating their success rates and training efficiency for temporal-logic tasks.

Designed and implemented the two-stage repair and planning framework in Python, including:

LTL-to-automaton translation and GR(1)-based realizability checks.

Symbolic repair operators that adjust task specifications while preserving user intent.

Interfaces to MoveIt for physical feasibility checking and feedback.

Developed ROS2 integration:

rclpy control nodes that map symbolic actions to joint-space trajectories.

State-monitoring modules that update the automaton based on execution results and sensor feedback.

Designed multi-scenario experiments (sequential tasks, safety-constrained tasks, and skill-deficit repair) and defined evaluation metrics such as task-completion rate, violation count, and planning time.

Results & Takeaways

Demonstrated a working prototype where a ROS2-simulated manipulator autonomously executes multi-step tasks directly from temporal-logic descriptions, without hand-coded task scripts.

Verified that the repair loop can turn initially unrealizable tasks into executable plans by modifying the logical specification rather than the low-level controller, improving transparency and explainability for human users.

Gained hands-on experience at the intersection of formal methods, motion planning, and robotics software engineering, including GR(1) synthesis, ROS2 middleware, MoveIt motion planning and asynchronous, feedback-driven control.

This project showcases my ability to move from theory (temporal logic, automata) to a full-stack robotic implementation, and to design systems that support intuitive, high-level human-robot interaction while remaining grounded in real-time motion control.

Robocon 2023 "Flower of Angkor"

In 2023 I joined the SCUT Robotics Lab competition team for the Robocon China contest themed "Flower of Angkor". The game required two large mobile robots to coordinate fast, high-precision tasks under strict time limits, combining rapid maneuvering, accurate throwing/placing mechanisms and reliable autonomous operation. As part of the mechanical subgroup, I contributed to the end-to-end design and realization of the competition robots, from early concepts to on-site debugging.

My responsibilities

Participated in concept design and task decomposition, translating the game rules into concrete mechanical functions (high-speed drive, climbing/positioning structures, and scoring mechanisms).

Built detailed 3D CAD models of chassis frames and functional modules, paying attention to stiffness, weight distribution, and maintainability for quick adjustments between matches.

Selected and sized motors, gearboxes and belt/chain transmissions, and designed mounting interfaces that respected electrical and control constraints.

Led part of the manufacturing and assembly work, including CNC-machined aluminum parts, laser-cut plates and 3D-printed fixtures; coordinated tolerance control so that mechanical and electronic subsystems could be assembled without rework.

Assisted with on-field testing and failure analysis, iteratively reinforcing weak structures, tuning clearances and improving accessibility for rapid repair during the competition.

Outcome & reflection

The final robot system ran stably throughout the national competition, and our team was awarded a National Third Prize. This experience significantly strengthened my abilities in system-level mechanical design, design-for-manufacturing, and cross-disciplinary collaboration under real competition pressure, and gave me first-hand insight into how high-performance robotic systems are engineered and debugged as a team.